Picus 2 connection python script example

Intro

This script example is “terminal-like python program”, based on Bleak Bluetooth library (from git). Bleak is recommended by our development team because of wide compatibility, and it is easy to find guides and examples for.

Requirements

- Python 3.8

- Bleak library (pip install bleak OR coda install -c conda-forge bleak)

Instructions (also provided by script)

1) python picus2_ble_example.py Picus-12345678                  (12345678 is to be replaced by pipette serial number)

-> ...

-> Connected, start typing and press ENTER... To disconnect press CTRL-C

2) {"data":"GET_VERSION"}

-> sent: b'{"data":"GET_VERSION"}\r\n'
-> received: bytearray(b'ACK\r\n')
-> received: bytearray(b'BEGIN\r\n') 
-> received: bytearray(b'CP-3.8\r\n')
-> received: bytearray(b'END\r\n')

3) <CTRL-C pressed>

-> Device was disconnected, goodbye.